A STUDY ON CRUMB RUBBER: OPPORTUNITIES FOR DEVELOPMENT OF SUSTAINABLE CONCRETE IN THE NEW MILLENNIUM

| First Author | First Author Affiliation (Designation) with Mobile No. |
| --- | --- |
| Second Author | Second Author Affiliation (Designation) with Mobile No. |
| Third Author | Third Author Affiliation (Designation) with Mobile No. |
| Name of Corresponding Author | Fourth Author Affiliation (Designation) with Mobile No. |

Subject  :

Full Address for Hard Copy With Name :

Mobile No.

1first.author@first.edu

2second.author@second.com

3third.author@third.edu

(Please specify here your complete correspondence address of the corresponding author)

Abstract-Crumb rubber is a term usually applied to recycled rubber from automotive and truck scrap tyres. During the recycling process steel and fluff is removed leaving Tyre rubber with a granular consistency. Continued processing with a granulator and/or cracker mill, possibly with the aid of cryogenics or mechanical means, reduces the size of the particles further. It is not possible to discharge the rubbers in the environment because they decompose very slowly and cause lots of pollution. So, it is necessary to have a relevant use of these wastages. These waste materials can be used to improve some mechanical properties of concrete. Addition of rubber to concrete results in the improvements of some mechanical and dynamical properties.Such as more energy absorption, better ductility and better crack resistance. By using the waste tyre (crumb rubber)one can reduce the harmful effect on environment and provide sustainable concrete.

Key words-crumb rubber, utilization, compressive strength, low cost, sustainable

INTRODUCTION

Utilization of industrial waste products in concrete has attracted attention all around the world due to the rise of environmental consciousness. Accumulations of stockpiles of Tyres are dangerous because they pose a potential environmental concern, fire hazards and provide breeding grounds for mosquitoes that may carry disease. Tyre pile fires have been an even greater environmental problem.

Tyre pile fires can burn for months, sending up an acrid black plume that can be seen for dozens of miles. That plume contains toxic chemicals and air pollutants, just as toxic chemicals are released into surrounding water supplies by oily runoff from Tyre fires. In order to prevent the environmental problem from growing, recycling Tyre is an innovative idea or way in this case. Recycling Tyre is the processes of recycling vehicles Tyres that are no longer suitable for use on vehicles due to wear or irreparable damage (such as punctures).

Tarun have reported that the compressive strength of rubberized concrete can be improve when fine aggregate was fully replaced by fine crumb rubber. He also indicated that if the rubberParticles have rougher surface or given a pretreatment, the better and improved bonding may develop with the surrounding matrix, and that may result in higher compressive strength.

Pitiel outlined that crumb rubber responses were found to denote greater flexibility andToughness with larger deflection at peak load, longer post-peak load responses and higher fracture energy.WasteTyres are a tremendous problem throughout the world. It is hardly surprising that in many countries it has been deduced that the best option is to simply burn them in cement kilns. At least in this way, the reasoning goes, some of the energy invested in the Tyre is reclaimed.

Figure 1: crumb rubber in cracker mill

Sources: www.googleimages .com/ crumb rubber

INDIAN TYRES INDUSTRY:

TABLE – 1

GENERAL DETAILS

| Consumption world ranking | 4th |
| --- | --- |
| Total number of Tyre Companies | 36 |
| Total number of TyreFactories | 51 |
| Tyre Production 2012-13 (Estimated) | 110 Million |
| Industry Turnover (Estimated) | Rs. 31000 crores |
| Capacity Utilization (Estimated) | 84% |
| Growth in Truck & Bus tyre production | 15% |

Source: Indian rubber industry statistics

APPLICATIONS OF WASTE TYRES IN CIVIL CONSTRUCTION

- Tyrerubberinconcreteandmortars

Research on cement-based products modified with Tyre rubber – such as concrete and mortar – has been carried out for many years in order to examine the potential utilisation of waste Tyres in concrete production. Waste Tyres have been used to partially replace the aggregates in mortars and concrete. Tyre rubber can be used to produce workable concrete for specific applications, provided that adequate selection processes are undertaken – including the amount, gradation and shape of Tyre particles. This section deals with the properties of either mortar or concrete modified with waste Tyre rubber.

CASE STUDY

In the present study, effect of crumb rubber as fine aggregate replacementonthecompressivestrength of   concrete having mixproportions of 1:1.31:1.14   was   investigated. Thepercentages of replacementswere0%, 10 %,20% and30%by weight of fine aggregate. Tests wereperformed forcompressive strength orall replacementlevels of crumb rubber at differentcuringperiods(7-days& 28-days).

CONCLUSIONS

We can say that for 1m3M20 grade of concrete consumption of fine aggregate is 775.96 kg. Here in specimen M-3 we replace fine aggregate by 24.62 kg of crumb rubber for 1m3M20 grades of concrete. So, we can say that up to 15% foundry sand utilized for economical and sustainable development of concrete.Uses of crumb rubberin concrete can reduce the harmfulness to the environment and produce a ‘greener’ concrete for construction. An innovative supplementary Construction Material is formed through this study.

REFERENCES:

[1]Aiello, M. A., and Leuzzi, F. (2010), “Waste Tyrerubberized concrete: Properties at fresh and hardenedstate.” Journal of Waste Management, ELSEVIER, 30,1696-1704.

[2]Batayneh, M. K., Marie, I., and Asi, I. (2008),“Promoting the use of crumb rubber concrete indeveloping countries.” Journal of Waste Management,ELSEVIER, 28, 2171-2176.

[3]Egyptian Code Committee 203, (2003), “Experimentalguide for testing of concrete materials.” Part 3 of theEgyptian code of practice for the design andconstruction of reinforced concrete structures.

[4]Eldin, N. N., and Senouci, A. B. (1993), “Rubber-Tyreparticles as concrete aggregate.”Journal of Material inCivil Engineering, ASCE, 5(4), 478-496.